# DIP PAPER REPORT

Global median filtering forensic method

based on Pearson parameter statistics

GROUP 2

MEMBERS:-

ABHAS BIND

JAIHIND PRATAP

VAIBHAV VERMA

# INTRODUCTION

Median filter:

Two-dimensional non-linear median filter replaces the central pixel value with the median of all pixel values in the neighbourhood window of size s × s. For images, s normally takes odd values (s = 3, 5, 7, ...). In this work, the most common median filtering window sizes s = 3 and s = 5 are considered. For a particular pixel with value p(m, n) at the position (m, n) in an image, the corresponding pixel after median filtering with window size s × s, ps(m, n) can be written as

ps(m, n) = medians[p(i, j)]

Where                                               i∈m-s-12,.......m+s-12

j∈n-s-12,.......n+s-12

and                                                     m, n, i, j ∈ Z

Here medians[ . ] denotes median operator which operates by first ranking grey levels of window size s × s in ascending order and then selecting the middle grey level among them.

Median filtering detection

In digital filtering, the median filter is often used to remove noise from an image or signal. Noise reduction is a common pre-processing step used to improve the results of subsequent processing (for example, edge detection on an image). Median filtering is very widely used in digital image processing because, under certain conditions, it preserves edges while removing noise (but see the discussion below), also having applications in signal processing.

Pearson parameter: κ

Many applications have been developed using the Pearson distribution system.

A Pearson system of distributions is characterised by the parameter κ which is defined as the polynomial ratio of skewness and kurtosis.A brief derivation of the parameter κ in terms of skewness and kurtosis is discussed for better understanding.

An appropriate method for generation of unimodal probability distributions can be achieved by solving the differential equation

1f(x)df(x)d(x) = x+ba0+a1x+a2x2………(1)

where g(x) is the probability density function (pdf) and b, a0, a1 and a2 are constant parameters. The constant parameters b, a0, a1 and a2  can be obtained in terms of skewness (S) and kurtosis (K). After that, (2) can be rewritten as (see (3)) . Here Su=μ3/μ23/2 and Ku=μ4/μ22. μ2, μ3 and μ4 are second, third and fourth centralised moments, respectively. The task is to obtain these constant parameters in terms of moments of the distrib- ution. We assume that density function falls rapidly such that xjf (x ) vanishes (for all j ≥ 0) at the end points, say (α, β). Multiplying (1) by xi and integrating by parts, we get

αβxi(a0+a1x+a2x2)df(x)d(x)d(x) = αβf(x)(x+b)xid(x)         …………(2)

On further simplification

We note that the first term in (3), vanishes at end points (α, β). Defining i th moment   μ'i= αβf(x)xid(x)   about origin we obtain

bμ'i+ia0μ'i-1+(i+1)a1μ'i +(i+2)a2μ'i+1= -μ'i+1         ……….(4)

The values of unknown parameters viz. b, a0, a1 and a2 can be determined from the solution of four simultaneous Eqs. (5–8) which are obtained for i = 0, 1, 2, 3. We choose

origin such that  μ'1= 0 , and note that μ'0 = 1. Defining the central moment

μi=αβf(x)(x-μ'1)id(x)

Gives

It may be noted that for Eq. (5) μ'i-1 is taken as 0. Solving

these equations, we get

Using (9-12) and introducing the Skewness (Su=μ3/μ23/2) and Kurtosis (Ku=μ4/μ22) and Eq(1)

...(15)

The nature of roots of the quadratic equation (a0+a1x+a2x2= 0) in (1) depends on the discriminant function (a12 − 4a0a2)≥ 0 which can be rewritten as a12/4a0a2≥ 1. It is instructive to introduce a parameter κ (κ = a12/4a0a2 ), which in terms of Skewness and Kurtosis becomes

κ vector andκ vector histogram

In image MFRs, a block with single residual value (SRV) is referred to as SRV block. These SRV blocks have zero variance, resulting in undefined skewness and kurtosis values. As a result, the κ values of SRV blocks are also undefined. The undefined κ values for SRV blocks are denoted as κSRV. However, SRV blocks do not participate in the construction of the final κ vector, but the count of such blocks in an image is used as a feature in this work. It has been observed that the location of SRV blocks in original image MFR and median filtered image MFR does not coincide. Thus, to maintain equal lengths of original image MFR κ vector and median filtered image MFR κ vector, only the blocks which do not have κSRV in either of κ vectors have been considered.

κ vectors construction from corresponding example blocks of size 6 × 6 (red marked blocks in Figs. 1b and e) from original image MFR (Fig. 1b) and 3 × 3 median filtered image MFR (Fig. 1e) have been demonstrated. For 6 × 6 sized example block, κ values for each of sliding blocks of size 3 × 3 has been determined and denoted as kopi where op ∈ {org3, m f 3} and i ∈ N where N = {n|n ∈ ℤ+ & 1 ≤ n ≤ 16}. Here, org3 and mf3 denote blocks from original image MFR and median filtered image MFR with s = 3, respectively. The blue coloured block in Fig. 1c represents an SRV block in the original image MFR example block and computed kSRVis at ninth position in korg3 vector. Corresponding ninth position in a kmf3 vector in Fig. 1f is also filled with blue colour. The green coloured block in Fig. 1f represents one of the SRV blocks in the 3 × 3 median filtered image MFR example block. Corresponding computed kSRV is at the third position in kmf3 vector.Other SRV blocks in 3 × 3 median filtered image MFR example block result in kmf35, kmf36 and kmf37 in kmf3 and filled with green colour. Corresponding locations in korg3vectors are also filled with green colour as shown in Fig. 1c. For the equal length of korg3 and kmf3 vectors, final κ vectors are determined by removing all green and blue coloured blocks (shown by red cross marks on green and blue colour filled blocks in Figs. 1c and f) from korg33 and kmf3 vectors. Resulting final vectors are termed as korg3fand kmf3fas shown in Fig. 2

...Fig2

# RESULTS

Figure 1 Original image

Figure 2. Gray-scale image of original and its MFR

Figure 3. Median filtered image of original and its MFR

Distinct residual value blocks of a image(ucid0001.tif) of the UCID database original images MFR (ORG_IMG_MFR) and median filtered images (s = 3 and s = 5) MFRs (MF3_IMG_MFR and MF5_IMG_MFR)

| Nr | Original image | MFR of median filtered image(s=3) | MFR of median filtered image(s=5) |
| --- | --- | --- | --- |
| 1 | 4.19 | 14.70 | 10.92 |
| 2 | 5.66 | 21.47 | 15.94 |
| 3 | 2.80 | 24.90 | 16.97 |
| 4 | 4.95 | 17.05 | 15.47 |
| 5 | 10.22 | 11.14 | 14.38 |
| 6 | 14.20 | 6.60 | 12.52 |
| 7 | 16.64 | 2.93 | 8.84 |
| 8 | 20.14 | 0.98 | 4.06 |
| 9 | 20.88 | 0.21 | 0.90 |

Average percentage (%) of distinct residual value blocks in the UCID database original images MFR (ORG_IMG_MFR) and median filtered images (s = 3 and s = 5) MFRs (MF3_IMG_MFR and MF5_IMG_MFR)

| Nr | Original image | MFR of median filtered image(s=3) | MFR of median filtered image(s=5) |
| --- | --- | --- | --- |
| 1 | 1.56 | 40.24 | 51.37 |
| 2 | 3.99 | 33.70 | 30.01 |
| 3 | 7.76 | 15.19 | 11.66 |
| 4 | 9.04 | 7.24 | 5.02 |
| 5 | 9.37 | 2.58 | 1.48 |
| 6 | 11.38 | 0.81 | 0.36 |
| 7 | 15.40 | 0.20 | 0.07 |
| 8 | 20.63 | 0.028 | 0.009 |
| 9 | 20.85 | 0.002 | 0.0007 |

Moment ratio of korg3fand  kmf3f for image(ucid0001.tif) of the UCID database

Reference:

https://ur.booksc.me/book/23196079/f536d4